Exp No:10a -Analysis of Critical Frequency and Maximum Usable Frequency (MUF) using MATLAB

Code:

clc;

clear;

close all;

%% EXPERIMENT 10a

% Analysis of Critical Frequency and Maximum Usable Frequency (MUF)

%% Objective 1: Critical Frequency Analysis

Nmax = 0.1:0.05:1.0;          % Electron Density (10^12 electrons/m^3)

fc = 9*sqrt(Nmax);             % Critical Frequency (MHz)

%% Objective 2: MUF Analysis

theta = 0:1:70;                % Angle of Incidence (degrees)

fc_fixed = 6.3;                % Critical Frequency (MHz)

MUF = fc_fixed*sec(deg2rad(theta));

%% Objective 3: Comparative Analysis

MUF_ratio = MUF/fc_fixed;

%% Display Results

fprintf('\nCRITICAL FREQUENCY AND MUF ANALYSIS\n');

fprintf('-------------------------------------\n');

fprintf('Critical Frequency = %.2f MHz\n',fc_fixed);

fprintf('MUF at 60 Degree = %.2f MHz\n', ...

fc_fixed*sec(deg2rad(60)));

%% Plot Results

figure('Name','Critical Frequency and MUF Analysis', ...

'NumberTitle','off');

% Graph 1: Critical Frequency vs Electron Density

subplot(2,2,1);

plot(Nmax,fc,'LineWidth',2);

grid on;

xlabel('Electron Density (10^{12} electrons/m^3)');

ylabel('Critical Frequency (MHz)');

title('Critical Frequency vs Electron Density');

% Graph 2: MUF vs Angle of Incidence

subplot(2,2,2);

plot(theta,MUF,'LineWidth',2);

grid on;

xlabel('Angle of Incidence (Degree)');

ylabel('MUF (MHz)');

title('MUF vs Angle of Incidence');

% Graph 3: MUF Ratio

subplot(2,2,3);

plot(theta,MUF_ratio,'LineWidth',2);

grid on;

xlabel('Angle of Incidence (Degree)');

ylabel('MUF/fc');

title('MUF Ratio');

% Graph 4: Comparison of Critical Frequency and MUF

subplot(2,2,4);

plot(theta,MUF,'LineWidth',2);

hold on;

yline(fc_fixed,'--','LineWidth',2);

grid on;

legend('MUF','Critical Frequency');

xlabel('Angle (Degree)');

ylabel('Frequency (MHz)');

title('Comparison of fc and MUF');

fprintf('\nSimulation completed successfully.\n');

Output:

Exp No:10b:Analysis of Skip Distance and Tropospheric Propagation using MATLAB

Code:

clc;

clear;

close all;

%% EXPERIMENT 10b

% Analysis of Skip Distance and Tropospheric Propagation using MATLAB

%% Objective 1: Skip Distance Analysis

h = 250;                       % Ionospheric Height (km)

theta = 10:1:70;               % Angle of Incidence (degrees)

Skip_Distance = 2*h*tand(theta);

%% Objective 2: Tropospheric Propagation

ht = 100;                      % Transmitter Height (m)

hr = 50;                       % Receiver Height (m)

LOS_Distance = 4.12*(sqrt(ht)+sqrt(hr));

%% Objective 3: Comparative Analysis

Coverage_Ratio = Skip_Distance/LOS_Distance;

%% Display Results

fprintf('\nSKIP DISTANCE AND TROPOSPHERIC PROPAGATION ANALYSIS\n');

fprintf('---------------------------------------------------\n');

fprintf('Ionospheric Height = %.2f km\n',h);

fprintf('Skip Distance at 60° = %.2f km\n', ...

2*h*tand(60));

fprintf('LOS Distance = %.2f km\n',LOS_Distance);

fprintf('Coverage Ratio = %.2f\n', ...

(2*h*tand(60))/LOS_Distance);

%% Plot Results

figure('Name','Skip Distance and Tropospheric Propagation', ...

'NumberTitle','off');

% Graph 1: Skip Distance vs Angle

subplot(2,2,1);

plot(theta,Skip_Distance,'LineWidth',2);

grid on;

xlabel('Angle of Incidence (Degree)');

ylabel('Skip Distance (km)');

title('Skip Distance vs Angle');

% Graph 2: Tropospheric LOS Distance

subplot(2,2,2);

bar(LOS_Distance);

grid on;

title('Tropospheric LOS Distance');

ylabel('Distance (km)');

set(gca,'XTickLabel',{'LOS Range'});

% Graph 3: Coverage Ratio

subplot(2,2,3);

plot(theta,Coverage_Ratio,'LineWidth',2);

grid on;

xlabel('Angle of Incidence (Degree)');

ylabel('Coverage Ratio');

title('Coverage Ratio Analysis');

% Graph 4: Sky Wave vs Tropospheric Propagation

subplot(2,2,4);

plot(theta,Skip_Distance,'LineWidth',2);

hold on;

yline(LOS_Distance,'--','LineWidth',2);

grid on;

legend('Skip Distance','LOS Distance');

xlabel('Angle (Degree)');

ylabel('Distance (km)');

title('Comparison of Sky Wave and Tropospheric Propagation');

fprintf('\nSimulation completed successfully.\n');

Output: