Exp No:9a Simulation of Ground Wave and Sky Wave Propagation using MATLAB

Code:

clc;

clear;

close all;

%% Experiment 9a: Ground Wave and Sky Wave Propagation

%% Objective 1: Ground Wave Propagation

Pt = 10;                 % Transmitted Power (kW)

d = 1:1:500;             % Distance (km)

% Ground Wave Electric Field Strength

E_ground = (300*sqrt(Pt))./d;

%% Objective 2: Sky Wave Propagation

h = 250;                 % Ionosphere Height (km)

theta = 60;              % Angle of Incidence (degrees)

fc = 5;                  % Critical Frequency (MHz)

% Skip Distance

Skip_Distance = 2*h*tand(theta);

% Maximum Usable Frequency (MUF)

MUF = fc*sec(deg2rad(theta));

%% Objective 3: Comparative Analysis

Sky_Range = Skip_Distance*ones(size(d));

%% Display Results

fprintf('\nGROUND WAVE AND SKY WAVE ANALYSIS\n');

fprintf('---------------------------------\n');

fprintf('Transmitted Power = %.2f kW\n', Pt);

fprintf('Ionospheric Height = %.2f km\n', h);

fprintf('Skip Distance = %.2f km\n', Skip_Distance);

fprintf('Maximum Usable Frequency (MUF) = %.2f MHz\n', MUF);

%% Plot Results

figure('Name','Ground Wave and Sky Wave Propagation', ...

'NumberTitle','off');

% Graph 1: Ground Wave

subplot(2,2,1);

plot(d,E_ground,'LineWidth',2);

grid on;

xlabel('Distance (km)');

ylabel('Field Strength (mV/m)');

title('Ground Wave Propagation');

% Graph 2: Maximum Usable Frequency

subplot(2,2,2);

bar(MUF);

grid on;

title('Maximum Usable Frequency');

ylabel('Frequency (MHz)');

set(gca,'XTickLabel',{'MUF'});

% Graph 3: Skip Distance

subplot(2,2,3);

bar(Skip_Distance);

grid on;

title('Skip Distance');

ylabel('Distance (km)');

set(gca,'XTickLabel',{'Sky Wave'});

% Graph 4: Sky Wave / Communication Range

subplot(2,2,4);

plot(d,Sky_Range,'LineWidth',2);

grid on;

xlabel('Distance (km)');

ylabel('Distance (km)');

title('Sky Wave Communication Range');

fprintf('\nSimulation completed successfully.\n');

Output:

---------------------------------------------------------------------------------------------------------------------

Exp 9b : Simulation of Space Wave Propagation Characteristics using MATLAB

Code:

clc;

clear;

close all;

%% EXPERIMENT 9b

% Simulation of Space Wave Propagation Characteristics using MATLAB

%% Objective 1: Space Wave Propagation

f = 900;                 % Frequency (MHz)

Pt = 30;                 % Transmit Power (dBm)

d = 1:1:100;             % Distance (km)

% Free Space Path Loss

FSPL = 32.44 + 20*log10(f) + 20*log10(d);

% Received Power

Pr = Pt - FSPL;

%% Objective 2: LOS and Radio Horizon

ht = 50;                 % Transmitter Height (m)

hr = 25;                 % Receiver Height (m)

% Radio Horizon Distance

dh_tx = 4.12*sqrt(ht);

dh_rx = 4.12*sqrt(hr);

% LOS Communication Range

d_LOS = 4.12*(sqrt(ht) + sqrt(hr));

%% Objective 3: Link Budget Analysis

Gt = 10;                 % Transmitter Gain (dB)

Gr = 8;                  % Receiver Gain (dB)

% Received Power using Link Budget

Pr_link = Pt + Gt + Gr - FSPL;

%% Display Results

fprintf('\nSPACE WAVE PROPAGATION ANALYSIS\n');

fprintf('---------------------------------\n');

fprintf('Operating Frequency = %.0f MHz\n',f);

fprintf('Transmitted Power = %.2f dBm\n',Pt);

fprintf('LOS Distance = %.2f km\n',d_LOS);

fprintf('Radio Horizon of Transmitter = %.2f km\n',dh_tx);

fprintf('Radio Horizon of Receiver = %.2f km\n',dh_rx);

fprintf('Received Power at 10 km = %.2f dBm\n',Pr(10));

%% Plot Results

figure('Name','Space Wave Propagation Characteristics', ...

'NumberTitle','off');

% Graph 1: Free Space Path Loss

subplot(2,2,1);

plot(d,FSPL,'LineWidth',2);

grid on;

xlabel('Distance (km)');

ylabel('FSPL (dB)');

title('Free Space Path Loss');

% Graph 2: Received Power

subplot(2,2,2);

plot(d,Pr,'LineWidth',2);

grid on;

xlabel('Distance (km)');

ylabel('Received Power (dBm)');

title('Received Power vs Distance');

% Graph 3: LOS Communication Range

subplot(2,2,3);

bar(d_LOS);

grid on;

title('LOS Communication Range');

ylabel('Distance (km)');

set(gca,'XTickLabel',{'LOS Range'});

% Graph 4: Link Budget Analysis

subplot(2,2,4);

plot(d,Pr_link,'LineWidth',2);

grid on;

xlabel('Distance (km)');

ylabel('Power (dBm)');

title('Link Budget Analysis');

fprintf('\nSimulation completed successfully.\n');

Output:

Model Graph: